Thermal and Nonthermal Inactivation of Foodborne Pathogens on Low-Moisture Foods: A Systematic Review

Norfaziey Aini Sukarno1 , Arthika Kalaichelvan4, Pichamon Limcharoenchat5, Noor Azira Abdul Mutalilb1,3 , Nurul Hawa Ahmad1, 2, 3*

Table 2 Thermal inactivation studies of low-moisture foods between 2001-2021

| Target microorganisms | Food sample |  | Thermal treatment |  | Processing parameters |  | Log reduction |  |  |  | Model |  |  | D-values (min) and z-values (℃) | Significant findings |  |  | References |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Salmonella spp. | Cookies |  | Baking Hot water bath |  | Temperature (60 - 70℃) |  | >5 log CFU/g |  |  |  | N/A |  |  | D75℃ = 11.9 – 62.3 Z – value: 14.5 – 15.8℃ | Baking hard cookies for at least 11.5 minutes at 185°C and soft cookies for at least 20.5 minutes at 165.6°C results in a 5-log reduction in Salmonella population. |  |  | (Channaiah et al., 2021) |  |
| Salmonella spp. Enterococcus faecium | Inshells hazelnut |  | Hot air assisted radio frequency (HARF) |  | Temperature (55 - 75℃) |  | 0.5 -5.6 log CFU/g |  |  |  | N/A |  |  | D55°C = 33.96 D60°C = 5.11 - 42.83 D65°C = 1.08 - 26.63 D70°C = 0.19 – 5.26 D75℃ = 1.21 Z -value = 5.23 – 7.44°C | By combining surface disinfectants/salt solution and RF process conditions, an efficient pasteurization protocol must be developed. |  |  | (Chen et al., 2021b) |  |
| Enterococcus faecium | Almond |  | Drying (column dryer) |  | Temperature (Ambient - 60℃) |  | < 1 log CFU/g |  |  |  | N/A |  |  | N/R | Harvested almonds need to be sorted and dehulled before drying to increase the drying efficiency and moisture uniformity. |  |  | (Chen et al., 2021a) |  |
| Salmonella Typhimurium Salmonella Senftenberg Salmonella Enteritidis PT30 | Almonds |  | Dry heating |  | Temperature (90 - 95℃) Time (15 - 60 mins) |  | S. Typhimurium: 0.17 - >6.04 log CFU/g S. Senftenberg: 1.33 - >5.2 log CFU/g S. Enteritidis PT30: 0.4 - >6.54 log CFU/g |  |  |  | N/A |  |  | N/R | The combination of dry heat and vacuum packaging significantly reduced Salmonella on almonds while causing no colour changes. |  |  | (Song and Kang, 2021) |  |
| Salmonella E. coli NRRL B-2354 | Dried basil leaves |  | Dry heating method |  | Water activity (0.40 - 0.70) Temperature (70 - 80℃) |  | N/R |  |  |  | Weibull model |  |  | Salmonella: D75°C = 3.30 to 9.14 min Enterococcus faecium: D75°C = 6.53 to 14.07 min | The kill ratio of surrogate to pathogen ranged from 1.4 to 2.8, indicating that it is a conservative surrogate for Salmonella for thermal pasteurisation validation. |  |  | (Verma et al.,2021a) |  |
| Salmonella spp. |  | Whole peppercorns |  | Ethylene oxide fumigation |  | Temperature (46 - 60℃) Time (2 – 180 mins) Relative humidity (30 – 50%) |  | 1.15 – 4.92 log CFU/g |  | Weibull model |  |  | N/R |  |  |  | Temperature and relative humidity have a significant impact on black peppercorn EtO fumigation. More than 5 log reductions of Salmonella can be reduced in 20 min at RH 50% and temperature 60°C used in this study. |  | (Wei, Chen, et al., 2021) |
| Salmonella Enteritidis PT-30 Salmonella Tennessee K4643 Salmonella Agona 447967 Enterococcus faecium NRRL B-2354 |  | Almond meal |  | Desiccation |  | Water activity (0.25 and 0.45) Temperature (22 - 95℃) |  | 0.8 – 1.5 log CFU/g |  | N/A |  |  | Salmonella spp.: D80℃ = 27.5 49.6 D85℃ = 13.7 – 42.8 D90℃ = 7.0 – 19.9 D95℃ = 4.6 – 9.3 Enterococcus faecium: D80℃ = 36.6 D85℃ = 19.5 – 76.4 D90℃ = 9.1 – 33.0 D95℃ = 5.8 – 14.9 |  |  |  | The thermal resistance of Salmonella remained unchanged after one year of storage especially at 4℃. |  | (Zhu et al., 2021) |
| Salmonella spp. | Cured egg yolk |  | Curing |  | Time (2 - 144 h) |  | < 5.6 log CFU/g |  | N/A |  |  | N/R |  |  |  | The cured egg yolk safety can be improved by 2 hours of curing time followed by temperature-controlled water circulation process. |  | (Lopes et al., 2020) |  |
| Salmonella Typhimurium ATCC 14028 | Peanut kernels |  | Desiccation and roasting |  | Storage days (0 – 180 days) Temperature (120°C) Time (0 – 30 mins) |  | 4- 5 log CFU/g |  | Weibull model |  |  | D120°C = 34.86 – 67.99 |  |  |  | Long-term storage of blanched peanut kernels caused desiccation stress, which increased S. Typhimurium ATCC 14028 thermal resistances. |  | (Pereira et al., 2020) |  |
| Enterococcus faecium NRRL B2354 | Peanut oil |  | Desiccation |  | Water activity (0.33 - 0.93) |  | 0 – 5 log CFU/g |  | N/A |  |  | D80℃ = 87 - 1539 |  |  |  | The key factor of thermal resistance of bacteria in oil is the equilibrium water activity of the bacteria-in-oil system. |  | (Yang et al., 2020) |  |

| Listeria monocytogenes | Non-fat dry milk (NFDM) | Ethylene glycol bath | Water activity (0.30) Temperature (80℃) Time (60 minutes) | 5-log reduction | N/A | D70℃ = 21.3 – 66.2 D75℃ = 9.4 – 33.5 D80℃ = 4.3 – 14.6 z-values = 14.62 - 15.95 °C | This represents a promising intervention strategy for improving the bactericidal efficacy of thermal treatment while maintaining milk powder quality. | (Ballom et al.,2019) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Salmonella Escherichia coli | Wheat flour | Dry bath | Temperature (70℃) Time (60 minutes) | Salmonella = 1.91 log CFU/g Escherichia coli: 4.22 log CFU/g | Weibull model | Salmonella: D70℃ = 17.4 min Enterohemorrhagic Escherichia coli: D70℃ = 0.2–1.6 min | The Weibull model fit the thermal death kinetics of Salmonella and E. coli O45, O121, and O145 in wheat flour well. | (Forghani et al.,2019) |

| E. coli O121 | Wheat flour | Desiccation | Temperature (70 - 80℃) | 0.64 – 6.95 log CFU/g |  | N/A | D70℃ = 18.16 D75℃ = 6.47 D80℃ = 4.58 Z- value = 14.57℃ |  | E. coli O121 was found to be least thermally resistant than Salmonella Enteritidis PT30 when assessed under the same environments and using the same methodology. | (Suehr et al., 2019) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Salmonella Enteritidis PT30 Salmonella Tennessee K464 Salmonella Agona 447967 Enterococcus faecium NRRL B- 2354 | Cocoa powder | Ethylene glycol bath | Temperature (70 - 80°C) | Salmonella spp.: 3.75 log CFU/g Enterococcus faecium: 1.39 log CFU/g |  | Weibull model | Salmonella spp.: D70°C: 31.6 – 46.2 D75°C: 16.9 – 20.5 D80°C: 7.0 – 11.5 E. faecium: D70°C: 25.8 – 59.9 D75°C: 13.7 – 28.9 D80°C: 4.7 – 16.1 Z- value: 13.0 – 17.5 |  | Water activity affects the thermal inactivation rates on both bacteria; thus, it should be considered as a significant factor of the thermal resistance of Salmonella in cocoa powder. | (Tsai et al., 2019a) |
| Listeria monocytogenes | Cocoa powder | Ethylene glycol bath | Temperature (70 - 80℃) | >4 log CFU/g |  | N/A | D70℃ = 21.9 Z – value = 14.9 – 15.5℃ |  | Listeria monocytogenes survived in cocoa powder for an extended period and was detectable after a 12-month RT storage period, and thermal resistance is related to aw of samples. | (Tsai et al., 2019b) |
| E. coli O26 E. coli O103 E. coli O111 E. coli O157 | Wheat flour | Dry bath | Temperature (55 - 70℃) | 1.66 – 4.06 log CFU/g |  | Weibull model | D70℃ value: E. coli O26 = 5.75 E. coli O103 = 8.67 E. coli O111= 8.10 E. coli O157 = 6.69 |  | The study indicates that the E. coli strains tested remained visible at room temperature and survive the treatment at 70℃ for 60 mins in wheat flour. | (Forghani et al., 2018) |
| Salmonella Typhimurium Salmonella Senftenberg Cronobacter sakazakii | Milk powder | Drying | Temperature (90 and 100℃) Time (30 and 120s) |  | 0.45 – 1.12 log CFU/g | N/A | N/R |  | The ratio of unculturable cells observed was strongly related to the loss of respiratory activity and weakly related to membrane permeability. | (Lang et al., 2018) |
| Listeria monocytogenes | Wheat flour | Ethylene glycol bath | Water activity (0.3-0.6) Temperature (70 - 80°C) |  | 2.5 log CFU/g | N/A | D70°C - 80 °C = 37.10 - 1.59 z-values = 12.9 - 9.9°C |  | This study also sheds light on the development of thermal inactivation strategies for L. monocytogenes and other foodborne pathogens in foods with similar matrices. | (Taylor et al., 2018) |
| Salmonella | Oat flour | Extrusion Processing | Moisture content (14 to 26%) Temperature (65 - 85℃) | 5.5 log CFU/g |  | Response surface model |  | N/R | The developed response surface model can be used to identify the extrusion process conditions to achieve the desired reduction of Salmonella based on the product's moisture and fat contents. | (Verma et al.,2018) |
| Salmonella enterica serovar Agona | Oat flour | Extrusion | Temperature (82°C) Water activity (0.89) | > 5-log CFU/g |  | N/A |  | N/R | Extrusion is a good way to reduce Salmonella because most processes used to make cereals and other low water activity foods exceed these parameters. | (Anderson et al.,2017) |
| Salmonella spp. | Wheat flour | Desiccation | Biofilm producing strains | 0.59 -1.49 log CFU/g |  | N/A |  | D80℃ = 3.1 – 21.7 | Salmonella thermal resistance in low-moisture foods like wheat flour was influenced by preformed biofilm. | (Villa-Rojas et al., 2017a) |
| Salmonella Enteritidis PT30 | Wheat flour | Desiccation Hydration | Water activity (0.3 and 0.6) | 2.5 – 4.3 log CFU/g |  | N/A |  | D80℃ = 1.33 – 7.32 | When applying thermal resistance data to industrial pasteurization validations, the response period to new water activity is negligible. | (Smith and Marks, 2015b) |
| Salmonella | Peanut Butter Cookies | Baking | Temperature = 177°C Water activity: 0.82 - 0.51 | After baking 13 - 14 min = 5.2 to 6.2 log reduction |  | N/A |  | N/R | Salmonella in peanut butter cookies can be reduced by 5 logs or more with proper baking. | (Lathrop et al.,2014) |

| Salmonella spp. | Whole grain oat flour | Twin screw extrusion | Fat (5–15%) Moisture content (14–26%) Temperature (55–85 °C) Screw speed (75–225 rpm) | 0.0 to 9.0 log CFU/g | N/A | N/R | Twin screw extrusion can be a useful process intervention step. | (Verma and Subbiah, 2019) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Salmonella Typhimurium ATCC 50115 Salmonella Braederup H9812 Escherichia coli ATCC 25922 | Red pepper powder | Heat blocking system | Water activity (0.55 – 0.86) Temperature (62 - 68℃) Particle size: 0.45 – 1.0mm | >1.75 log CFU/g | Weibull model Bigelow model | D62℃ = 18.28 D65℃ = 10.10 D68℃ = 6.37 | Lower temperature is effective against higher water activity of red pepper powder to kill Escherichia coli ATCC 25922. | (Zhang et al., 2020a) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Salmonella Senftenberg 775 W | Non-fat dry milk (NDM) and hydrated NDM | Heating | aw of hydrated NDM = 0.991 aw of NDM = 0.199 Temperature 65°C | 4.3 – 4.4 log CFU/g | N/A | N/R | To avoid the chance of selecting an incorrect target Salmonella serovar for thermal inactivation studies of low-aw milk powders, Salmonella Senftenberg 775 W should be used within a cocktail, not as a single serovar. | (Sekhon et al.,2021) |
| Salmonella spp. | Hazelnuts Cocoa shells | Heating | Temperature (100 and 120℃) Time (0 – 30 mins) | 0 - 6.93 log CFU/g | N/A | D100℃ = 2.56 – 11.29 Z- value = 11.85 – 17.36 | Increasing the moisture content of dry materials (10% w/w) before or during heating significantly reduces microbial heat resistance, resulting in lower temperatures and shorter times required to destroy Salmonella. | (Izurieta and Komitopoulou, 2012) |
| Pediococcus acidilactici Salmonella spp. Enterococcus faecium | Toasted oat cereals Peanut butter | Heating block Hot water bath | Temperature (63 - 95°C) | 1.1 – 2.9 log CFU/g | Weibull model | N/R | Pediococcus acidilactici ATCC 8042 has the potential to serve as a Salmonella surrogate and create additional data on Enterococcus faecium NRRL-B2354 in two low water activity foods, toasted oat cereal and peanut butter. | (Deen and Diez-Gonzalez, 2019) |

| Salmonella H9812 E. coli ATCC 25922 Staphylococcus aureus ATCC 25923 | Walnuts shells | Heating block system | Temperature (50 - 64℃) Time (1.5 – 15 mins) Water activity (0.586 - 0.931) Moisture content (8.93 - 18.10%) | Salmonella: 3.45 – 4.25 log CFU/g E. coli: 2.84 – 3.96 log CFU/g Staphylococcus aureus: 1.23 – 3.0 log CFU/g | Weibull model |  | Staphylococcusaureus (aw= 0.586): D64℃ = 10.21 D67℃ = 5.57 D70℃ = 2.52 | The heating block system was ideal for investigating the effect of heating rates and aw on microorganism thermal resistance. | (Zhang et al., 2018) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Salmonella spp. | Whole milk powder Non-fat dairy milk | Hot air assisted radiofrequency | Temperature (85 - 95℃) Time (15 – 120 min) | 0.42 - >5 log CFU/g | N/A |  | N/R | Using HARF to preheat milk powders to target pasteurisation temperatures and then holding in a hot air oven for the desired time effectively pasteurises WMP and NFDM at a water activity of 0.10. | (Wei et al., 2021) |
| Salmonella spp. Enterococcus. faecium NRRL B- 2354 | Apple pieces | Hot air drying | Temperature (104 and 135°C) Time (0 – 127.5 mins) | Salmonella spp.: 0.5 – 7.4 log CFU/g Enterococcus faecium: 0.2 – 7.1 log CFU/g | N/A | N/R |  | At the temperatures tested, E. faecium inactivation was slower than Salmonella, thus it would be a good surrogate for in-plant validation studies. | (Grasso-Kelley et al., 2021) |
| Listeria monocytogenes Salmonella spp. Enterococcus faecium NRRL B-2354 E. coli | Pistachios | Hot oil Hot water | Temperature (80 - 121℃) Time (30 – 90s) | L. monocytogenes: 1.2 – 3.26 log CFU/g Salmonella spp.: 0.76 – 3.1 log CFU/g E. faecium: 0.67 – 1.92 log CFU/g E. coli: 1.08 – 4.81 log CFU/g | Weibull model | N/R |  | In the pilot-scale oven, Enterococcus faecium NRRL B-2354 outperformed Salmonella in terms of thermal resistance across a wide range of treatments, treatment times, and temperatures. | (Moussavi et al., 2020) |
| Salmonella spp. | Pecans nutmeats | Hot peanut oil | Temperature (110 - 138℃) Time (0 - 4 mins) | 4.67 – 4.91 log CFU/g | N/A | N/R |  | The temperatures and times used to oil roast nuts are sufficient to reduce Salmonella by 5 log CFU/g. | (Beuchat and Mann, 2011a) |
| Salmonella enterica E. coli O157:H7 L. monocytogenes Enterococcus faecium | In shells pecans | Hot water | Temperature (70 - 90℃) Time (1 – 5 mins) | Salmonella enterica: 1.79 – 6.59 log CFU/g E. coli O157:H7: 0.9 – 7.02 log CFU/g Listeria spp.: 4.6 – 5.49 log CFU/g Enterococcus faecium: 0.9 – 5 log CFU/g | N/A | D70℃ = 1.15 – 1.72 D80℃ = 0.83 – 1.19 D90℃ = 0.41 – 0.92 Z- value = 49.57 – 76.22℃ |  | The hot water treatment showed great promise when used as a kill step to reduce the risk of foodborne pathogens during the post-harvest processing of in-shell pecans. | (Kharel et al., 2018) |

| Salmonella Enteritidis PT 30 ATCC 1045 | Almonds | Hot water and oil | Temperature (88 - 127℃) | >5 log CFU/g |  | Weibull model |  | D88℃ = 12.7 Z value = 88℃ | Hot oil treatments are more effective compared to hot water treatment. |  | (Mohammad et al., 2020b) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enterococcus faecium | Peanut oil | Oil bath | Temperature (80℃) Relative humidity (0 – 100%) Oil layers (0.1, 0.26 and 0.51) | 0.24 – 0.65 log CFU/g |  | Weibull model |  | D80℃ = 1.3 - 4764 | According to this study, oil as a barrier to moisture diffusion is another mechanism that causes oil's protective effect on bacteria from thermal inactivation. |  | (Yang et al., 2021) |
| Salmonella Enteritidis PT30 | Wheat flour | Oil bath | Temperature (80℃) Time (10 - 60 mins) | 0.8 – 5.7 log CFU/g |  | N/A |  | N/R | Shows how desiccation affect the Salmonella cell’s structure that might clarify the mechanism of survival in LMF |  | (Xu et al., 2020) |
| Enterococcus faecium NRRL B-2354 Salmonella Enteritidis PT 30 | Wheat flour | Oil bath | Temperature (75 - 85℃) Water activity - – 0.6) Moisture content (10 – 14%) |  | N/R | Weibull model |  | Salmonella: D75℃: 12.0 – 24.5 D80℃: 4.2 – 11.4 D85℃: 1.1 – 5.9 E. faecium: D75℃: 25.5 – 65.8 D80℃: 11.4 – 35.4 D85℃: 2.7 – 15.9 Z value: 10.2 – 16.9℃ | Enterococcus faecium has higher D-values and equivalent z-values than Salmonella Enteritidis, indicating that it is a good surrogate for thermal process validation in wheat flour. |  | (Liu et al., 2018b) |
| Salmonella Agona | Wheat flour | Oil bath | Temperature (80℃) Water activity (0.5 – 0.9) Fat and protein content (0.5 – 0.98) |  | N/R | Weibull model Bigelow model |  | D80℃ = 1.3 - 4764 | There was a correlation between temperature and aw based on the amount of fat or protein, indicating that these factors must be considered when predicting the thermal inactivation of Salmonella in foods. |  | (Yuqiao et al., 2018) |
| Salmonella spp. L. monocytogenes Enterococcus faecium | Confectionary Seasoning Chicken meat powder | Oil bath | Temperature (70 - 140℃) Time (1.5 – 2 mins) |  | Salmonella spp.: 2.0 – 3.0 log CFU/g L. monocytogenes: 1.7 – 3.4 log CFU/g Enterococcus faecium: 2.3 – 4.4 log CFU/g | Weibull model |  | Salmonella spp.: D80℃ = 9.51 – 67.27 L. monocytogenes: D80℃ = 7.99 – 11.81 Enterococcus faecium: 24.95 – 126.2 | 5 log of Salmonella, L. monocytogenes or E. faecium can be inactivate with heating to 111.2, 105.3 or 111.8°C respectively. |  | (Rachon et al., 2016) |
| Enterococcus faecium NRRL B-2354 Salmonella Enteritidis PT30 | Almond kernels | Moist air oven | Temperature (121- 177℃) Water activity (0.24 – 0.78) Humidity (0 – 70%) |  | 4.4 log CFU/g | Bigelow model |  | N/R | The study highlights the significance of methodology, aw, and process humidity when validating thermal pasteurization processes for LMF, which should assist processors in selecting and interpreting validation methods. |  | (Jeong et al., 2017) |
| Salmonella Enterococcus faecium | Chia seeds | Pasteurization | Water activity (0.53) Temperature (80°C - 90.8°C). | N/R |  | Log linear or Weibull Bigelow model |  | N/R | Salmonella and Enterococcus faecium thermal inactivation kinetics on chia seeds could be used to develop thermal pasteurisation processes for chia seeds. |  | (Lau et al., 2021) |
| Salmonella spp. | Milk powder | Pasteurization | Milk type (Nonfat and whole milk) Fat content (0.62 and 29.46) Water activity - – 0.3) Temperature (75 - 85℃) | >0.4 log CFU/g |  | Bigelow model |  | D75℃ = 16.95 – 41.1 D80℃ = 8.11 – 22.83 D85℃ = 3.64 – 10.11 | Salmonella thermal resistance increases as the water activity decreases. The Bigelow model can be used to design a thermal pasteurization system for milk powders. |  | (Wei et al., 2020a) |
| Salmonella Oranienburg | Cocoa beans | Roasting | Temperature (100 - 150°C) Time (2 – 100 mins) | 2.8 - >5 log CFU/g |  | Weibull model |  | D100°C: 33.34 D110°C: 18.7 D115°C: 12.92 D120°C: 10.50 D130°C: 4.20 D140°C: 1.9 Z-value: 32.0°C | The results indicate that a 10-minute roasting at 150°C can result in a reduction of more than 5 log10 CFU/bean. |  | (Yan et al., 2021) |
| Salmonella Typhimurium Salmonella Newport Salmonella Enteritidis Salmonella Tennessee | Sunflower seeds | Roasting | Temperature (107.2 - 135℃) Time (5 – 45 mins) | >4 log CFU/g |  | N/A |  | N/R | Only after roasting the sunflower seeds for 45 minutes at 135℃ have saleable water activities, resulting in a.7-log reduction in Salmonella. |  | (Kottapalli et al., 2020) |
| Bacillus cereus Geobacillus stearothermophilus | Cocoa beans and nibs | Roasting | Temperature (110 - 140℃) Time (70 – 600 mins) | Bacillus cereus: 4 – 4.6 log CFU/g Geobacillus stearothermophilus: 4.5 – 5.3 log CFU/g |  | N/A | Bacillus cereus: D110℃ = 92.2 - 149 D125℃ = 30.5 - 56 D140℃ = 17.2 -23 Geobacillus stearothermophilus: 110℃ = 105 - 184 D 125℃ = 39.6 - 76 D140℃ = 30 - 49 |  |  | The use of 140°C during roasting with low spore-forming bacteria populations can improve roasting efficiency in terms of microbial inactivation. | (Paula et al., 2019) |
| Salmonella spp. | Tahini | Roasting | Storage days (119 days) Temperature (95 - 130°C) Time (0 – 90 mins) | 0 – >4 log CFU/g |  | N/A | D90°C = 24.7 D130°C = 15.0 |  |  | Over 119 days, no change in Salmonella populations was observed in tahini. These findings highlight the critical importance of aw during the roasting step, and Salmonella that survives roasting is likely to survive the tahini's in RT. | (Zhang et al., 2017) |
| Salmonella spp. | Cocoa beans and nibs Conching of milk chocolate | Roasting | Temperature (110 - 140℃) | 0.83 – 5.39 log CFU/g |  | N/A |  | D110℃ = 4.79 – 8.86 D120℃ = 3.62 – 8.02 D130℃ = 2.55 – 3.75 D140℃ = 2.47 – 2.5 |  | The results showed that the type of matrix, process temperature, and initial count all influenced Salmonella resistance. | (Nascimento et al., 2012) |
| Salmonella spp. Listeria monocytogenes Escherichia coli | Whole peppercorns | Steam pasteurization | Water activity (0.35 – 0.69) Temperature (70 and 75℃) | 1.85 - >5 log CFU/g |  | N/A |  | N/R |  | Microbial load can be reduced when it is mild steam at 75℃ and still maintaining the visual quality of the black peppercorns. | (Zhou et al., 2019) |
| E. coli spp. Salmonella spp. Listeria spp. | Almonds Pistachios | Steaming | Temperature (100 - 200℃) Time (1 – 30s) | E. coli spp.: 3.0 – 6.2 log CFU/g Salmonella spp.: 2.7 – 6.5 log CFU/g Listeria spp.: 2.7 – 5.7 log CFU/g |  | N/A |  | E. coli spp. D100℃ = 5.28 – 9.88 Salmonella spp. D100℃ = 4.87 – 11.15 Listeria spp. D100℃ = 6.68 – 11.12 |  | Superheated steaming is an effective method for inactivating foodborne pathogens on almonds and pistachios while preserving product quality. | (Ban and Kang, 2016) |
| Enterococcus faecium | Peanut butter | Superheated steam | Water activity (0.19 - 0.80) Temperature (125 - 250℃) | > 5 log CFU/g |  | Weibull models |  | D125℃ = 129.70 - 6.33s D175℃ = 32.41 - 3.38s D225℃= 24.62 - 1.93s D250℃= 18.49 - 3.22s Z value = 194.66℃ | E. faecium inactivation kinetics on peanut butter-contaminated surfaces can provide detailed information for superheated steam sanitation treatment, which can be applied to environmental surfaces for effective microbial inactivation. |  | (Park et al., 2021) |
| Salmonella Typhimurium Salmonella Enteritidis | Black peppercorns Pecans Almonds | Superheated steam | Temperature (100 -180℃) Time (1 – 45 s) | >6 log CFU/g |  | Weibull model |  | D100℃ = 4.65 – 9.2 Z – value = 47.06 – 146.26℃ | Salmonella was completely inactivated on food tested in 40 s, regardless of whether the SS or SHS treatment was used. The SS treatment was ineffective because the moisture content of the samples was high. |  | (Ban et al., 2018) |
| Salmonella Typhimurium Salmonella Enteritidis | Black peppercorns Pecans Almonds | Superheated steam | Temperature (100 -180℃) Time (1 – 45 s) | >6 log CFU/g |  | Weibull model |  | D100℃ = 4.65 – 9.2 Z – value = 47.06 – 146.26℃ | Salmonella was completely inactivated on food tested in 40 s, regardless of whether the SS or SHS treatment was used. The SS treatment was ineffective because the moisture content of the samples was high. |  | (Ban et al., 2018a) |
| Salmonella Enteritidis PT30 | Glucose Fructose Honey powder | TCA | Temperature (80℃) Water activity (0.3 – 0.4) Moisture content (0.02 – 0.5%) | >4 log CFU/g |  | N/A |  | D80℃ = 9.7 – 63.8 | During thermal processing, the D-values of Salmonella Enteritidis PT30 in high sugar-low aw products decreased with increasing aw, following a semi-log linear trend. |  | (Alshammari et al., 2020b) |
| Salmonella spp. | Burger binder Skim milk powder | TDT cells | Temperature (70℃) Time (5 - 25 mins) | < 2 log CFU/g |  | Weibull model |  | Burger: D70℃ = .5.57 – 27.42 | Salmonella spp. thermal resistance was determined by strain type, inoculation methodology, aw, and the nature of the food components. |  | (Dhaliwal et al., 2021) |
| Salmonella spp. Listeria spp. Enterococcus faecium NRRL B- 2354 | Desiccated shredded coconut | TDT cells | Temperature (75 - 90℃) Water activity (0.25 – 0.45) | >5 log CFU/g |  | N/A |  | Salmonella spp.: D80℃ = 38.7 – 53.2 Listeria spp.: 14.2 – 40.2 Enterococcus faecium NRRL B- 2354: D80℃ = 49.6 – 85.5 | In DSC, Listeria monocytogenes has less thermal resistance than Salmonella. The study focuses on thermal inactivation strategies for controlling Salmonella and Listeria monocytogenes during the DSC post-drying process. |  | (Dhowlaghar et al., 2021) |
| Salmonella spp. Enterococcus faecium | Egg powder | TDT cells | Temperature (20 - 80℃) Time (0 - 150 mins) Water activity (0.3 – 0.74) | Salmonella spp.: 0 - >6 log CFU/g Enterococcus faecium: 0 - >7 log CFU/g |  | N/A |  | Salmonella spp.: D80℃ = 5.1 – 25.9 Enterococcus faecium: D80℃ = 10.4 – 43.8 | The D values for both bacteria had a linear relationship with the water activity and the composition differences. |  | (Pérez-Reyes et al., 2021a) |
| Salmonella spp. | Whole black peppercorns | TDT cells | Temperature (60 - 85℃) Water activity (0.33 – 0.75) | 0.41 – 2.19 log CFU/g |  | Weibull model |  | D75℃ = 106 - 198 | Modifying the food water activity may be an effective way to achieve the required level of pathogen activation in a relatively short period of time at low treatment temperatures. |  | (Marco et al., 2021) |
| Salmonella Enteritidis PT30 | Ground cinnamon | TDT cells | Temperature (70 - 80℃) | <1 – 4 log CGU/g |  | N/A |  | D70℃ = 3.3 – 31.3 Z – value: 11.9 – 18.2℃ | Milder thermal treatments can be used to control Salmonella in cinnamon powder, as well as possibly other spices or herbs containing antimicrobial compounds, for better product quality retention. |  | (Xie et al., 2021) |
| Listeria monocytogenes | Almond meal | TDT cells TWA | Temperature (70 - 85℃) Water activity (0.25 – 0.45) | N/R |  | N/A |  | D75℃, aw0.25 = 47.6 D75℃, aw0.45 = 17.5 D80℃, aw0.25 = 22.0 D80℃, aw0.45 = 11.0 Z – value: 15.4℃ | The log-linear model fit the inactivation kinetics of L. monocytogenes in almond meal well; Thermal resistance of L. monocytogenes in almond meal was found to be inversely related to sample aw. |  | (Zhu et al., 2020) |
| Salmonella spp. | Soy protein powder | TDT cells | Temperature (60 - 95℃) Water activity (0.13 – 0.82) | >5 log CFU/g |  | N/A |  | D80℃ = 4.0 – 155.2 | Salmonella thermal resistance is affected by treatment temperature and the water activity of the food environment. |  | (Tsai et al., 2019a) |
| Salmonella Enteritidis PT30 | Wheat flour Almond flour Whey protein | TDT cells | Water activity (0.25 – 0.8) | N/R |  | N/A |  | D20°C = 0.8 – 27.3 | The almond flour would need to be fully exposed to 80°C for 120 minutes to achieve a 6-log reduction of Salmonella Enteritidis PT30. |  | (Xu et al., 2019) |
| E. coli ATCC 25922 | Almond powder | TDT cells | Temperature (75℃) | 0.8 – 2.55 log CFU/g |  | Weibull model |  | D75℃ = 12.6 – 20.5 | The effects of storage environment on reducing bacterial populations and D75℃ of E. coli became more noticeable as storage temperature increased. |  | (Cheng and Wang, 2018) |
| E. coli spp. | Confectionary Seasoning Chicken meat powder | TDT cells | Storage days (1 – 28 days) Temperature (80 - 110°C) | Confectionary: 0.71 log CFU/g Seasoning: 1.48 log CFU/g Chicken meat powder: 1.15 log CFU/g |  | Weibull model |  | Confectionary D80°C = 97.36 Seasoning: D80°C = 29.89 Chicken meat powder: D80°C = 184.48 | The food matrix composition on STEC heat resistance should be considered to increase the effectiveness of STEC. Increasing the temperature from 80 - 120°C significantly higher level of STEC inactivation. |  | (Daryaei et al., 2018) |
| Salmonella spp. Enterococcus. faecium NRRL B- 2354 | Milk powder | TDT sandwich | Milk powder (Whole milk powder, nonfat dry powder) Temperature (85 - 95°C) | Salmonella spp.: 0.2 – 0.32 log CFU/g Enterococcus faecium: 0.21 – 0.33 log CFU/g |  | Weibull model |  | Salmonella spp.: D85°C: 7.98 – 10.98 D90°C: 3.35 – 4.80 D95°C: 1.68 – 1.82 E. faecium: D85°C: 16.96 – 19.81 D90°C: 7.9 – 9.72 D95°C: 4.16 – 4.94 | Salmonella and E. faecium thermal resistance did not change appreciably over the 90-day storage of Whole milk powder and nonfat dry powder. |  | (Wei et al., 2021a) |
| Salmonella spp. Enterococcus faecium NRRL B- 2354 | Black peppercorns | TDT sandwich | Water activity (0.25 – 0.65) Temperature (65 - 85°C) | Salmonella spp.: 0.2 – 0.5 log CFU/g Enterococcus faecium: 0.21 – 0.4 log CFU/g |  | Bigelow model |  | Salmonella spp.: D65°C: 13.3 – 46.0 D70°C: 3.9 – 20.5 D75°C: 2.0 – 43.1 D80°C: 17.6 D85°C: 6.2 Enterococcus faecium: D70°C: 12.0 – 34.1 D75°C: 3.3 – 67.6 D80°C: 1.5 – 28.9 D85°C: 12.6 | The D- values of Salmonella and E. faecium in ground black pepper were strongly influenced by water activity. Lower water activity resulted in better thermal resistance of Salmonella and E. faecium in ground black pepper. |  | (Wei et al., 2021b) |
| Bacillus spp. | Mesquite flour | Thermal treatment | Temperature (100 - 150℃) Time (30 – 120 mins) | 1.32 – 4.76 log CFU/g |  | N/A |  | D100℃ = 0.28 – 0.63 h D110℃ = 0.14 – 0.49 h | The findings shows that the natural occurring Bacillus spp. In flour are heat resistance, whereas increasing the water activity will increase the heat sensitivity. |  | (Fan et al., 2021) |
| Escherichia coli Salmonella Typhimurium Salmonella Senftenberg Cronobacter sakazakii | Milk powder | Thermocycler | Temperature (85 - 100℃) Water activity (0.11 – 0.58) | N/R |  | Weibull model |  | E. coli: D85℃ = 60.5 -139 Salmonella Typhimurium: D85℃ = 42.5 - 127 Salmonella Senftenberg: D85℃ = 56.4 - 156 Cronobacter sakazakii: D85℃ = 54 – 122 Z – value: 29.6 – 62.2℃ | The values obtained for Z value shows that Salmonella Typhimurium is the most resistant to temperature variations while E. coli is the most resistant to variation of water activity. |  | (Lang et al., 2017) |
| Salmonella Tennessee Salmonella Oranienburg | Peanut butter | Thin metal plate | Temperature (25 and 85℃) | 4 – 5 log CFU/g |  | N/A |  | Salmonella Tennessee: D85℃ = 11.95 Salmonella Oranienburg: D85℃ = 12.83 | Both of the serovars significantly survived in peanut butter at 25℃ when grown at solid media compared to culture in liquid. |  | (Keller et al., 2012) |
| Salmonella Enteritidis PT 30 Enterococcus faecium NRRL B-2354 ATCC 8459 Escherichia coli O157:H7 | Flaxseed Quinoa Sunflower kernels Black peppercorns | Vacuum pasteurization | Temperature (75 - 105℃) Time (0.5 - 5.0 mins) |  | ~7 – 8 log CFU/g | N/A |  | N/R | Pathogens were reduced by more than 5 logs in whole flaxseed, sunflower kernels, and peppercorns at 75 °C and milled flaxseed and quinoa at 85 °C. |  | (Shah et al., 2017) |
| Salmonella spp. Shiga toxin-producing Escherichia coli L. monocytogenes Pediococcus acidilactici | Apricot halves Macadamia nuts Raisins | Vacuum steam pasteurization | Temperature (62 - 82℃) Time (0 – 5 mins) |  | >5 log CFU/g | Weibull model |  | Salmonella spp.: D72℃ = 0.8 – 7.5 E. coli: D72℃ = 0.8 – 5.4 L. monocytogenes: D72℃ = 0.7 – 7.3 P. acidilactici: D72℃ = 1.1 – 10.3 | Depending on the temperature, low-temperature, vacuum-assisted steam pasteurisation provides a strategy for improving the safety of dried fruits and nuts in relatively short time periods. |  | (Acuff et al., 2020) |
| Salmonella serovars (Agona, Enteritidis, Montevideo, and Tennessee) | Flaxseeds | Vacuum steam pasteurization | Water activity (0.5) Temperature (71℃) |  | N/R | Geeraerd-tail model |  | D71°C = 1.0 to 1.5 | The amount of time stored prior to heat treatment would have little effect on the amount of time required to inactivate Salmonella on flaxseed. |  | (Malekmohammadi et al.,2020) |
| Aerobic bacteria count Yeast and molds | Whole and milled flax seeds | Vacuum steam pasteurization | Temperature (75 - 105℃) |  | Aerobic bacteria count: 0 – 2.7 log CFU/g Yeast and molds: <1 log CFU/g | N/A |  | N/R | When vacuum steam pasteurisation is used, the total number of aerobes, yeast, and moulds is reduced. The thermal treatment has no discernible effect on the chemical properties. |  | (Shah et al., 2018) |
| Salmonella spp. Enterococcus faecium | Whole peppercorns Cumin seeds | Vacuum steam pasteurization | Temperature (177℃) |  | Salmonella spp.: 1.92 – 1.93 E. faecium: 1.64 – 2.3 | Weibull model |  | N/R | In a short period of time, vacuum assisted steam pasteurization of spices was effective in reducing Salmonella and Enterococcus faecium may be used as a surrogate for inactivation on whole peppercorns and cumin seeds. |  | (Newkirk et al., 2018) |
| E. coli O121 Salmonella Enteritidis PT30 | Hard red spring in wheat flour | Vacuum steaming | Temperature (65 - 85℃) Time (4 and 8 mins) |  | Salmonella: 3.21 log CFU/g E. coli: 3.57 log CFU/g | N/A |  | N/R | In the flour milling industry, vacuum steaming can be an effective pathogen inactivation process. |  | (Snelling et al., 2020) |
| Listeria monocytogenes Salmonella spp. Enterococcus faecium NRRL B-2354 | Peanut butter Powder infant formula Wheat flour | Water bath | Temperature (60 - 90℃) Water activity (0.11 – 0.45) Fat content (2 – 45%) |  | N/R | N/A |  | Listeria monocytogenes: D75℃ = 9 – 17.9 Salmonella spp.: D75℃ = 4.7 – 21.1 Enterococcus faecium NRRL B-2354: D75℃ = 10.3 – 28.1 Z- value: 11.8 – 29.9℃ | Controlled heat treatments on organisms in inoculated powder infant formula showed the greatest thermotolerance, whereas the same organisms in wheat flour showed the least. |  | (Quinn et al., 2021) |
| Salmonella | Dry and hydrated non-fat dry milk (NFDM) and whole milk powder (WMP) | Water bath | Temperature (80 - 90°C) |  | 1.8 log CFU/g | N/A |  | D80 - 90°C = 18.9 - 4.4 min D80 - 90°C = 29.4 - 6.5 min D80 - 90°C = 17.9 - 4.4 min z-value = 6.4 – 16.3ºC | Salmonella population was reduced significantly in milk powders and corresponding hydrated powders during the storage period. |  | (Sekhon et al.,2021) |
| E. coli Listeria innocua, Lactobacillus plantarum Bifidobacterium lactis Enterococcus faecium | Wheat flour | Water bath | Moisture level (8 and 13%) Water activity (0.25 and 0.55) Temperature (82 and 87°C) |  | >5 log CFU/g | N/A |  | N/R | A heat treatment at 82°C for 5 minutes was effective in activating STEC in flour with a water content of 13%. To achieve the same level of inactivation at lower water content levels, a higher temperature or longer holding times would be required. |  | (Daryaei et al., 2020) |
| Salmonella spp. | Philippine flowing-type peanut butter | Water bath | Temperature (60 - 80℃) |  | 0.1 – 3.07 log CFU/g | N/A |  | D80℃ = 16.15 Z – value: 44.5℃ | According to the findings of this study, achieving safety against pathogenic microorganisms such as Salmonella enterica necessitates a relatively harsh thermal process. |  | (Pelaez et al., 2020) |
| Enterococcus faecium NRRL B-2354 | Almond meal | Water bath | Temperature (80℃) Inoculation method (Wet (WI), dry (DI), wet talc (WT) and powder talc (PT) |  | WI: 2.5 – 3.0 log CFU/g DI: 1.6 – 1.9 WT: 2.3 – 2.8 PT: 1.9 – 2.5 | N/A |  | D80℃: WI: 40.5 DI: 63.5 WT: 46.9 PT: 20.6 | Carriers like talc should not be used for inoculation of LMF unless the impact on the target organism is identified. |  | (Ahmad et al., 2019) |
| Salmonella | Almonds | Water bath | Temperature (80℃) Water activity (0.36) |  | N/R | Weibull model |  | D80℃ = 15.7 - 18.0 min | The sorption state of LMF products should be considered when estimating bacterial inactivation kinetics, developing models, and validating processes. |  | (Garces-vega et al.,2019) |
| Salmonella Enteritidis PT30 | Almond flour Wheat flour Date paste | Water bath | Temperature (80℃) |  | Almond flour: 0.42 – 1.13 log CFU/g Wheat flour: 0.42 – 0.98 log CFU/g Date paste: 0.32 – 0.70 log CFU/g | Weibull model |  | Almond flour: D80℃ = 18.3 – 49.8 Wheat flour: D80℃ = 8.9 – 19.5 Date paste: D80℃ = 1.2 – 3.5 | These findings suggest that the method of inoculation has a significant impact on the thermal resistance of Salmonella in LMF. |  | (Limcharoenchat et al., 2018) |
| Salmonella Coronobacter sakazakii Pediococcus acidilactici | Galactooligosaccharide | Water bath | Temperature (70 - 85°C) Time (0 – 150s) |  | Salmonella: 1.3 – 2.2 log CFU/g C. sakazakii: 2.2 – 2.7 log CFU/g P. acidilactici: 0.8 – 1.8 log CFU/g | N/A |  | Salmonella: D70°C = 0.24 Coronobacter sakazakii: D70°C = 0.2 P. acidilactici: D70°C = 1.72 Z – value: 13.04 – 33.18 | Pathogenic bacteria such as Salmonella and Coronobacter sakazakii may be resistant to heat treatments used to sterilize GOS during production. |  | (Bang et al., 2017) |
| Salmonella Enteritidis PT30 | Wheat flour | Water bath | Temperature (70 - 85℃) Water activity (0.3 – 0.7) | 0.26 – 1.01 log CFU/g |  | Weibull model Bigelow model |  | D75℃ = 2.28 – 13.37 | Water activity is a key factor in the thermal inactivation of Salmonella in LMF and should be accounted for in thermal inactivation models for these systems. |  | (Smith et al., 2016) |
| Salmonella Enteritidis PT30 | Wheat flour Peanut butter | Water bath | Temperature (80℃) | 2.75 – 5.07 log CFU/g |  | N/A |  | D80℃ = 9.7 – 63.8 | Changes in water activity at high temperatures, as well as the thermal resistance of Salmonella in all-purpose wheat flour and peanut butter. |  | (Syamaladevi et al., 2016) |
| Salmonella spp. | Peanut butter products | Water bath | Temperature (70 - 90℃) Different formulation (Fat, carbohydrate, protein, and sodium) | 1.01 – 5.86 log CFU/g |  | Weibull model |  | N/R | Salmonella in reduced fat peanut butter spread was significantly more thermally resistant than Salmonella in other peanut butter products. |  | (Li et al., 2014) |

| Salmonella Enteritidis PT30 | Almond kernels | Water bath | Temperature (56 - 80℃) Water activity (0.6 – 0.95) Moisture content (6 – 18%) | N/R | Weibull model | D68℃ = 0.42 – 6.97 Z – value: 8.28℃ | A minimum 4-log reduction, a 12-minute treatment at 60°C, and aw of 0.946 could be used to meet the USDA pasteurisation standard. | (Villa-Rojas et al., 2013) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Salmonella spp. | In shells pecans | Water bath | Temperature (75 – 95℃) Time (2 -15 mins) | 1.38 – 7.68 log CFU/g | N/A | N/R | To achieve a 5-log reduction on and in immersion-inoculated, dried in-shell, higher temperatures, or longer periods of exposure to hot water, or both, will be required. | (Beuchat and Mann, 2011b) |
| Salmonella enterica Escherichia coli O157:H7 | Peanut Butter | Water bath | Water activity (0.4 - 0.7) Temperature (72 - 90°C) | < 2- 7.1-log CFU/g | Bigelow model | Salmonella enterica: D72°C = < 30 min D90°C = < 10 min | The high carbohydrate content of peanut butter, combined with the low incubation temperature, resulted in higher bacterial survival during storage but lower bacterial resistance to heat treatment. | (He et al.,2011) |

Table 3 Nonthermal inactivation studies of low-moisture foods between 2001-2021

| Target microorganisms |  | Food sample |  | Nonthermal treatment | Processing parameters | Log reduction |  | Model |  | D-values (min) and z-values (℃) |  | Significant findings |  | References |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Salmonella ## Listeria monocytogenes ## Enterococcus faecium ## Listeria innocua |  | Cocoa powder |  | Radiofrequency heating | Water activity (0.45) Temperature (75 °C) | 4.6 log CFU/g |  | Bigelow model |  | L. innocua: D75°C = 3.6 z-values = 14.3◦C Salmonella: D90°C = 2.5 Enterococcus faecium: D90°C = 2.3 |  | Enterococcus faecium and L. innocua were appropriate surrogate strains for controlling Salmonella and L. monocytogenes, respectively, during RF processing of cocoa powder. |  | (Ballom et al., 2021) |  |
| ## Salmonella |  | Raw almonds |  | Pulsed light | One-time dipping of almonds in water for 1 min followed by a PL treatment of 500 g of almonds at an intensity of 0.75 W/cm2 for 18 min | >5 log reduction |  | N/A |  | N/R |  | Pasteurization of raw almonds using PL treatment in conjunction with prior water dipping could be a viable option. |  | (Liu et al., 2021) |  |
| ## E. coli O157:H7 Salmonella Typhimurium |  | Black pepper |  | Radiofrequency | Temperature (100°C) | E. coli O157: H7: > 6 log CFU/g Salmonella: > 6 log CFU/g |  |  | N/A | N/R |  | E. coli O157: H7 and S. Typhimurium were effectively inactivated by RF without affecting the quality of black pepper kernels. |  | (Wei et al., 2019) |  |
| ## Salmonella Enterococcus faecium NRRL B-2354 |  | Dried basil leaves |  | Radiofrequency heating | Temperature (100°C) | Salmonella:4.8 log CFU/g E. faecium: 2.7 log CFU/g |  |  | N/A | N/R |  | The rapid heating of the dried basil leaves by RF processing improves food safety while having no effect on quality. |  | (Verma et al., 2021b) |  |
| Salmonella Enterococcus faecium NRRL B-2354 |  |  | Black peppercorn Cumin seeds | Gaseous chlorine dioxide technology | ClO2 gas at 15 mg/L concentration and relative humidity 80% at room temperature (25°C) | Salmonella:>5 log CFU/g E. faecium: Black peppercorn: 4.36 log CFU/g Cumin seeds: 4.17 log CFU/g |  | Bigelow models |  | Salmonella: Black peppercorn - D25°C: 60.3 Cumin seeds D25°C - 58.7 |  | During the ClO2 treatment, Enterococcus faecium was discovered to be a suitable surrogate for Salmonella. |  | (Wei et al., 2021) |  |
| Salmonella |  |  | Whole milk powder, Non-fat Dry Milk | Hot air-assisted radiofrequency processing | Water activity (0.10) Temperature (95°C) | > 5 log reduction | N/A |  |  | N/R |  | This study validated a HARF-assisted thermal process for pasteurization of milk powder based on previously collected microbial inactivation kinetics data. |  | (Wei et al., 2021) |  |
| Aspergillus flavus |  |  | Peanut kernels | Radiofrequency | Temperature (65 - 70°C) | 3- >7 log CFU/g | Kopelman model |  |  | D68°C (aw= 0.74) = 8.3 D62°C (aw=0.88) = 5.7 |  | The non-uniformity of RF treatment causes the least A. flavus death at cold spot. The COMSOL software may aid in the development of RF. |  | (Zhang et al., 2021) |  |
| ## Salmonella enterica Enterococcus faecium |  |  | Cumin seeds | Continuous radio frequency | Belt speed: 28.2 m/h Cold spot Temperature: 99.6°C | S.enterica:: >5.52 log CFU/g E. faecium: > 6.52 log CFU/g | N/A |  |  | N/R |  | The use of RF in the industrial pasteurisation of cumin seeds has a high potential. |  | (Chen et al., 2020) |  |
| E. coli ATCC 25922 |  |  | Almond kernels | Radiofrequency heating | Controlled Atmosphere (2% O2, 20% CO2, 78% N2) Moisture content (8%) Temperature (72-78°C) | 4 log CFU/g | N/A |  |  | D75°C: 5.00 – 5.5 |  | Under CA, RF heating has the potential to be an effective treatment method for controlling E. coli ATCC 25922 in almond kernels and could be extended to pasteurisation applications. |  | (Cheng et al., 2020) |  |
| Salmonella enterica Enterococcus faecium |  |  | Egg white powder | Radiofrequency | Temperature (80℃) Time (0 - 16 h) | 0.58 - >5 log CFU/g | N/A |  |  | N/R |  | Enterococcus faecium was found to be a suitable surrogate for Salmonella. The validated RF-assisted thermal process has the potential to be scaled up for use in the egg industry. | (Wei et al., 2020b) |  |  |
| ## Enterococcus faecium NRRL B-2354 |  |  | Wheat flour | Radiofrequency | Water activity (0.45) Temperature (80 – 85°C) | 2.5–3.7 log CFU/g | Bigelow model |  |  | D80 °C = 8.3 min z-value =11.7 °C |  | A relatively slow RF heating rate with a hot-air assisted system is beneficial for improving RF temperature uniformity and obtaining a uniformly. | (Xu et al., 2020) |  |  |
| Salmonella Typhimurium |  | Red pepper powder |  | Radiofrequency | Water activity (0.44 – 0.70) | ≈ 4 log CFU/g |  | N/A |  | N/R | The absence of Salmonella Typhimurium sublethal injury cells (SICs) in red pepper powder with an initial aw of 0.44 was confirmed by these findings. |  | (Zhang et al., 2020b) |  |  |
| ## Cronobacter sakazakii ATCC 29544 |  | Powdered infant formula milk (PIFM) |  | Thermostatic radiofrequency | Water activity (0.2 – 0.4) Temperature (55 – 70 °C) | 0.16 log |  | N/A |  | D70°C = 23.3 min Z value = 14.90°C | RF has a high potential for use in the production of PIFM to achieve higher pasteurisation efficiency. |  | (Zhang et al., 2020b) |  |  |
| ## Salmonella spp. |  | Soft wheat flour |  | Radiofrequency | Temperature (80 - 90°C) | 7-log CFU/g |  | N/A |  | N/R | For SWF pasteurisation, the optimum RF-assisted thermal processing conditions of 80°C for 7 h and 90°C for 2 h were recommended, with no compromise in quality or functionality. |  | (Boreddy et al., 2019) |  |  |
| Salmonella enterica Enterococcus faecium NRRL B-2354 |  | Cumin seeds |  | Radiofrequency heating | Water activity (0.7419) | Salmonella: > 5.8 log CFU/g Enterococcus faecium: > 6.4 log CFU/g | N/A |  |  | N/R | Enterococcus faecium is a suitable surrogate of Salmonella in cumin seeds for RF microbial inactivation. Batch variation requires stricter process control parameters for RF microbial inactivation. |  |  | (Chen et al., 2020) |  |
| ## E. coli ATCC 25922 |  | Almond kernels |  | Radiofrequency (RF) assisted hot air heating system under modified atmosphere packaging | Moisture content (8.0%) Temperature (75°C) | 4-log reduction | N/A |  |  | D75 °C = 4.5 min | MAP pre-storage thermal treatments induced by RF energy have the potential to be an effective and environmentally friendly method of controlling E. coli ATCC 25922 in almond kernels. |  |  | (Cheng and Wang, 2019) |  |
| ## Bacillus cereus spores | Red pepper powder |  |  | Radiofrequency | Water activity (0.70) Temperature (90°C) | 4-log reduction | Weibull model |  |  | D90 °C = 5.8 min Z-value (°C) = 64.3 | RF inactivation effects could be improved by control the sample initial water activity level. |  |  |  | (Jiao et al., 2019) |  |
| ## Cronobacter sakazakii | Powdered infant formula milk |  |  | Radiofrequency assisted traditional thermal processing | Water activity (0.22) Temperature (65°C) | 5-log CFU/g | N/A |  |  | N/R | Due to lower lipid oxidation and a much shorter processing time, RF assisted-TTP is better suited for PIFM pasteurisation than TTP. |  |  |  | (Lin et al., 2020) |  |
| Salmonella enterica Enteritidis PT30 Enterococcus faecium NRRL B-2354 | Corn flour |  |  | Radiofrequency Freezing treatment | Water activity (0.45) Temperature (85°C) | Salmonella Enteritidis PT30: 6.59 log CFU/g and Enterococcus faecium:4.9log CFU/g | N/A |  |  | Salmonella Enteritidis PT30: D 85°C = 2.03 | For validation studies in packaged corn flour, E. faecium could be used as a surrogate. The results confirmed that RF in conjunction with freezing storage could significantly reduce the survival of both microorganisms in corn flour. |  |  |  | (Ozturk et al., 2019) |  |
| ## Staphylococcus aureus ATCC 25923 | In-shell walnuts |  |  | Radiofrequency | Moisture content (15.01%) | 4-log reduction | N/A |  |  | N/R | RF treatments could be considered as an effective method to control pathogens on in-shell walnuts. |  |  |  | (Zhang et al., 2019) |  |
| Salmonella Typhimurium | Red pepper powders |  |  | Radiofrequency heating | Water activity (0.71) Temperature (70°C) | > 5 log CFU/g | Weibull mode |  |  | N/R | Increasing initial aw could first increase log reductions and then decrease the log reductions, optimum aw level was 0.71 for RF inactivation of Salmonella in red pepper powders. |  |  |  | (Hu et al., 2018) |  |
| ## Aspergillus flavus | Peanut kernel |  |  | Radiofrequency | Water activity (0.72 - 0.92) | N/R | Weibull model |  |  | D62℃ = 43.48 D68℃ = 13.68 D74℃ = 2.77 | Least log reduction achieved when the RF heating is non uniform. |  |  |  | (Zhang et al., 2018) |  |
| ## Salmonella enterica Enterococcus faecium |  | Black peppercorn |  | Radiofrequency heating | Water activity (0.60) | Salmonella:5.31 log CFU/g Enterococcus faecium: 5.26-log CFU/g | N/A |  |  | N/R | RF heating is a promising thermal inactivation treatment for Enterococcus faecium appears to be a suitable surrogate for Salmonella to validate the efficacy of RF heating of black peppercorn without significant quality deterioration. |  |  |  | (Wei et al., 2018) |  |
| Enterococcus faecium NRRL B-2354 |  | Wheat flour |  | Radiofrequency | Temperature (75°C - 85°C) | 1.0 – 4.9 log CFU/g | N/A |  |  | D75°C = 14.27, D80°C = 5.92, D85°C = 2.79 Z-value = 13.1°C. | RF treatment is an efficient of inactivating target microorganisms in flour samples. Freeze-dried Enterococcus faecium proved to be an effective Salmonella surrogate in LMF. |  |  |  | (Liu, et al., 2018a) |  |
| ## Escherichia coli AW1.7 |  | Dry cells |  | High-Pressure Carbon Dioxide | Temperature (35°C) | >3 log reduction (CFU/ml) | N/A |  |  | N/R | Liquid and supercritical CO2 were ineffective in reducing the cell counts of dry E. coli isolates, and the effectiveness of gaseous CO2 was related to the diffusivity of CO2. |  |  |  | (Chen et al., 2017) |  |
| ## Salmonella Enteritidis PT 30 Enterococcus faecium NRRL B-2354 |  | Wheat flour |  | Radiofrequency pasteurization | Water activity (0.45) Temperature (85°C) | > 5 log CFU/g | N/A |  |  | S. Enteritidis: D85°C = 2.92 z-values: 12.8°C z-values of E. faecium = 11.7°C | Despite having similar z-values, Enterococcus faecium was more heat-resistant than Salmonella Enteritidis. |  |  |  | (Liu et al., 2018a) |  |
| ## Salmonella Enterococcus faecium |  | Organic wheat flour |  | Radiofrequency | Water activity (0.25) | Salmonella: 5 log CFU/g Enterococcus faecium: 3 log CFU/g | N/A |  |  | N/R | RF appears to be an acceptable method for pasteurising in wheat flour, and E. faecium B-2354 may be an adequate surrogate for future larger-scale evaluations of RF inactivation. |  |  |  | (Villa-Rojas et al., 2017a) |  |
| ## Aspergillus parasiticus |  | Corn grains |  | Radiofrequency heating | Moisture content (15.0% w.b) Temperature (70 °C) | 5-6 log CFU/g | N/A |  |  | N/R | To control Aspergillus parasiticus and maintain acceptable corn quality, RF treatments can provide an effective and rapid heating method. |  |  |  | (Zheng et al., 2017) |  |
| ## Cronobacter sakazakii Salmonella |  | Non-fat dry milk |  | Radiofrequency dielectric heating | Temperature (90°C) | 3-log CFU/g | N/A |  |  | Cronobacter sakazakii: D90°C = 5.37 - 5.57 Salmonella: D90°C = 4.55 – 5.82 z-values of Cronobacter sakazakii = 23.77 - 26.28°C z-values of Salmonella spp. = 26.92 - 20.92°C | Radio-frequency dielectric heating can be used to achieve target temperatures for NDM post-process lethality treatment before packaging in a faster and more uniform manner. |  |  |  | (Michael et al., 2014) |  |
| Salmonella spp. Enterococcus faecium NRRL B-2354 |  | Paprika White pepper Cumin powder |  | Radiofrequency heating | Temperature (70 - 80°C) | 4 – 5 log CFU/g |  | Weibull model |  | Salmonella: D70°C: 6.55- 16.61 D75°C: 3.09 – 8.01 D80°C: 1.21 – 4.47 E. faecium: D70°C: 9.62 – 28.31 D75°C: 4.51 – 18.64 D80°C: 1.82 – 9.53 Z- value: 13.6 – 19.1℃ | RF heating could be used as an alternative pasteurization method for spices, and it is feasible to design RF pasteurization processes using Enterococcus faecium and the F-value. |  |  |  | (Ozturk et al., 2020) |  |
| Salmonella Typhimurium Escherichia coli O157:H7 Staphylococcus aureus Listeria monocytogenes |  | Cardamom |  | X-ray | 350 Gy | 2 log CFU/g |  | Weibull models |  | E. coli: D10: 71.43 Gy tR values: S. Typhimurium: 53.57 L. monocytogenes: 87.74 S. aureus: 114.64 Gy | Pathogens in dry cardamom could be effectively inactivated using a 150 KeV low-energy X-ray. |  |  |  | (Zhang et al., 2021) |  |
| Salmonella Enteritidis PT30 |  | Almonds Dates Wheat |  | X-ray | Water activity (0.25 - 0.65) Time (7 - 103 weeks) | N/R |  | N/A |  | D10-value = 0.378 - 2.34 kGy | Product structure (whole, meals, powder, or paste), water activity (0.25 to 0.65 aw), and storage period (0 to 103 weeks) should all be considered when determining the efficacy of X-ray irradiation for inactivating Salmonella in various low-water-activity foods. |  |  |  | (Steinbrunner et al., 2019) |  |